Experiment / Assignment / Tutorial No. ___2___

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

Batch: B3 Roll No.: 121 Experiment / assignment / tutorial No.: 2

Title: Binary Adders and Subtractors

_____________________________________________________________________________________

Objective: To implement half and full adder–subtractor using gates and IC 7483

______________________________________________________________________________

Expected Outcome of Experiment:

CO2: Use different minimization technique and solve combinational circuits, synchronous & asynchronous sequential circuits.

______________________________________________________________________________

Books/ Journals/ Websites referred:

- VLab Link: http://vlabs.iitb.ac.in/vlabs-dev/labs/dldesignlab/experimentlist.html

- R. P. Jain, “Modern Digital Electronics”, Tata McGraw Hill

- M .Morris Mano, “Digital Logic & computer Design”, PHI

- http://physics.niser.ac.in/labmanuals/sem5/elect/7_ADDER%20SUBTRACTO  R%20CIRCUITS.pdf

Pre Lab/ Prior Concepts:

Adder: Addition of two binary digits is most basic operation performed by the digital computer. There are two types of adder:

- Half adder

- Full adder

Half Adder: Half adder is combinational logic circuit with two inputs and two outputs. It is the basic building block for addition of two single bit numbers.

Full adder: A half adder has a provision not to add a carry coming from the lower order bits when multi bit addition is performed. for this purpose a third input terminal is added and this circuits is to add A,B,C where A and B are the nth order bits of the number A and B respectively and C is the carry generated from the addition of (n-1) order bits. This circuit is referred to as full adder.

Subtractor: Subtraction of two binary digits is one of the most basic operations performed by digital computer .there are two types of subtractor:

- Half subtractor

- Full subtractor

Half subtractor: Logic circuit for the subtraction of B from A where A,B are 1 bit numbers is referred to as half subtract or .the subtract or process has two input and difference and borrow are the two outputs.

Full subtractor: As in the case of the addition using logic gates, a full subtractor is made by combining two half-sub tractors and an additional OR-gate. A full subtractor has the borrow in capability (denoted as BORIN) and so allows cascading which results in the possibility of multi-bit subtraction.

IC 7483

For subtraction of one binary number from another, we do so by adding 2’s complement of the former to the latter number using a full adder circuit.

IC 7483 is a 16 pin, 4-bit full adder. This IC has a provision to add the carry output to transfer and end around carry output using Co and C4 respectively.

2’s complement: 2’s complement of any binary no. can be obtained by adding 1 in 1’scomplement of that no.

e.g.  2’s complement of +(10)10 =1010is

| 1C of 1010 |  |  | 0101 |
| --- | --- | --- | --- |
|  |  | + | 1 |
| -(10)10 |  |  | 0110 |

In 2’s complement subtraction using IC 7483, we are representing negative number in 2’s complement form and then adding it with 1st number.

Implementation Details:

Half Adder Block Diagram

Half Adder Circuit

Truth Table for Half Adder

| Inputs |  | Outputs |  |
| --- | --- | --- | --- |
| A | B | S | C |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

From the truth table (with steps):

Full Adder Block Diagram

Full Adder Circuit

Truth Table for Full Adder

| A | B | CIN | S | COUT |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

From the truth table (with steps):

Half Subtractor Block Diagram

Half Subtractor Circuit

Truth Table for Half Subtractor

| A | B | DIFFERENCE(D) | BORROW(Bo) |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |  |
| 0 | 1 | 1 | 1 |  |
| 1 | 0 | 1 | 0 |  |
| 1 | 1 | 0 | 0 |  |

From the truth table (with steps) :

Full Subtractor Block Diagram

Full Subtractor Circuit

Truth Table for Full subtractor

| A | B | BIN | D | BOROUT |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

From the truth table (with steps):

IC 7483

Procedure:

- Locate the IC 7483 and 4-not gates block on trainer kit.

- Connect 1st input no. to A4-A1 input slot and 2nd (negative) no. to B4-B1 through 4-not gates (1C of 2nd no.)

- Connect high input to Co so that it will get added with 1C of 2nd no. to get 2C.

- Connect 4-bit output to the output indicators.

- Switch ON the power supply and monitor the output for various input combinations.

Example:

| 1) 710 -210 = 510 |  |  |
| --- | --- | --- |
| 7 |  | 0111 |
| 2 |  | 0010 |
| 1’C of 2 |  | 1101 |
|  |  | + 1 |
| 2’C of 2 |  | 1110 |

0111 +   1110 1 0101

Pin Diagram IC7483

Adder

Subtractor

Conclusion:

Thus, in this experiment, the symbols and circuits for half-adders, full-adders, half-subtractors and full-subtractors was learnt. Further, it was learnt that the IC 7483 processor can perform full addition as well as full subtraction. Thus, it is a useful component in any logic circuit.

Post Lab Descriptive Questions

- What is difference between half and full adder, half and full subtractor?

Ans.

| Sr. No. | Half Adder | Full Adder | Half Subtractor | Full Subtractor |
| --- | --- | --- | --- | --- |
| 1. | There are two inputs, A and B. | There are three inputs, A, B and Cin (input carry) | There are two inputs, A and B. | There are three inputs, A, B and Bin (input borrow). |
| 2. | Its logical expression is S = A ⨁ B and C = A.B | Its logical expression is S = A ⨁ B ⨁ Cin and Cout = Cin.(A⨁B) + A.B | Its logical expression is D = A⨁B and Bo = A.B | Its logical expression is D = Bin⨁A⨁B and Bout = Bin.(A⨁B)+A.B |
| 3. | It consists of one EX-OR Gate and one AND Gate. | It consists of two EX-OR Gates, two AND Gates and one OR Gate. | It consists of one EX-OR Gate, one NOT Gate and one AND Gate. | It consist of two EX-OR Gates, two NOT Gates, two AND Gates and one OR Gate. |
| 4. | The generated output is Sum and Carry from the input of two bits. | The generated output is Sum and Carry from the input of three bits including input Carry. | The generated output is Difference and Borrow from the input of two bits. | The generated output is Difference and Borrow from the input of three bits including input Borrow. |

- Perform the following Binary subtraction with the help of appropriate ICs:

- 7-5

- 5-7

- 9-4

Ans.